M.TECH VLSI & Embedded Systems IIOT Practical Exam 2023-2024

Name: Akash Kumar

PRN: 1032233339

Subject: Industrial Internet of Things

Date: 11/12/23

Aim: Traffic Light Control System

Objectives:

- Build a cyber physical traffic light control system using arduino or nodemcu nodes at each intersection

- Utilize nodemcu for communication between intersections and a raspberry pi as the centralized control unit. upload traffic data to thingspeak

- Implement a relay based actuation system for traffic lights and an LED countdown timer

CODE:

#include <ESP8266WiFi.h>

#include <ThingSpeak.h>

// Replace with your network credentials

const char* ssid     = "MITWPU-CIAP";

const char* password = "november2023";

// ThingSpeak Settings

unsigned long myChannelNumber = 2376023;

const char * myWriteAPIKey = "TVSHQLX7B53DVYE3";

// Define the relay pins

int redRelay = D1;

int yellowRelay = D2;

int greenRelay = D3;

WiFiClient client;

void setup() {

// Connect to Wi-Fi

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

}

// Set the relay pins as output

pinMode(redRelay, OUTPUT);

pinMode(yellowRelay, OUTPUT);

pinMode(greenRelay, OUTPUT);

// Initialize ThingSpeak

ThingSpeak.begin(client);

}

void loop() {

// Control the traffic lights and send data to ThingSpeak

// Red light for 10 seconds

digitalWrite(redRelay, HIGH);

ThingSpeak.writeField(myChannelNumber, 1, 1, myWriteAPIKey); // Send '1' to ThingSpeak

delay(10000);

digitalWrite(redRelay, LOW);

// Green light for 10 seconds

digitalWrite(greenRelay, HIGH);

ThingSpeak.writeField(myChannelNumber, 2, 1, myWriteAPIKey); // Send '1' to ThingSpeak

delay(10000);

digitalWrite(greenRelay, LOW);

// Yellow light for 2 seconds

digitalWrite(yellowRelay, HIGH);

ThingSpeak.writeField(myChannelNumber, 3, 1, myWriteAPIKey); // Send '1' to ThingSpeak

delay(2000);

digitalWrite(yellowRelay, LOW);

}

CODE for connection with raspberry PI:

import socket

import requests

# Create a socket

s = socket.socket()

# Define the host and port

host = 'localhost'

port = 12345

# Bind the socket

s.bind((host, port))

# Listen for connections

s.listen(5)

while True:

# Accept a connection

c, addr = s.accept()

# Receive data from the NodeMCU

data = c.recv(1024)

# Close the connection

c.close()

Interfacing and Output:

:

RED SIGNAL

GREEN SIGNAL

YELLOW SIGNAL

OUTPUT on Thingspeak:

Conclusion: I have successfully implemented 2 objectives from given statement and hence completed my practical exam.